Qualcomm Staff Engineer

Interview Preparation Guide

Part 2 of 3: Interrupt Handling, Device Drivers, Platform Bring-Up,

TrustZone/QSEE, Yocto, Debugging, Power Management & Qualcomm-Specific Topics

| Field | Details |
| --- | --- |
| Candidate | Sandeep Kumar |
| Target Role | Staff Engineer — Embedded Linux / Qualcomm SoC |
| Series | Part 2 of 3 |
| Experience | 12 Years (ARM/ARM64, Kernel, SoC Platforms) |
| Document | Interrupt Handling • Device Drivers • Bring-Up • TrustZone • Yocto • Debug • Power • SMEM/QMI • Coding |

PART 2 — QUICK REFERENCE: KEY SECTIONS

| Section | Topic |
| --- | --- |
| Section 1 | C/C++ Fundamentals & Data Structures |
| Section 2 | Interrupt Handling & ARM GICv3 |
| Section 3 | Device Drivers: I2C, SPI, PCIe, USB |
| Section 4 | Qualcomm Platform Bring-Up (UART, TLMM, Boot Sequence) |
| Section 5 | TrustZone / QSEE Architecture |
| Section 6 | Yocto Project & Build System |
| Section 7 | Debugging Tools: JTAG, ftrace, perf, QDSS |
| Section 8 | Power Management (Runtime PM, RPMH) |
| Section 9 | Qualcomm-Specific: SMEM, QMI |
| Section 10 | Coding Questions (Bit Manipulation, Linked List) |
| Section 11 | Interview Process & Priority Topics |

SECTION 1: C/C++ FUNDAMENTALS & DATA STRUCTURES

Q1 What is the difference between volatile, const volatile, and restrict in C?

volatile

Tells the compiler the variable can change outside program control (hardware register, ISR, shared memory). Prevents optimization — every access goes to memory.

volatile uint32_t *reg = (volatile uint32_t *)0x40001000;

*reg = 0x01;          /* write goes to hardware register */

uint32_t val = *reg;  /* read always fetches from hardware */

const volatile

Read-only from software perspective, but hardware can change it. Used for read-only status registers.

const volatile uint32_t *status_reg = (const volatile uint32_t *)0x40001004;

/* Software cannot write to status_reg, but HW may change it any time */

restrict

C99 keyword — tells the compiler that a pointer is the only way to access that memory in its scope. Enables aggressive optimization (no aliasing). Used in DSP/SIMD code.

void copy(uint8_t * restrict dst, const uint8_t * restrict src, size_t n) {

/* Compiler may vectorize — guaranteed no overlap between dst and src */

while (n--) *dst++ = *src++;

}

➡ Chain Q: Why is volatile not sufficient for SMP synchronization?

Because volatile only prevents compiler reordering, not CPU out-of-order execution. On SMP (multi-core ARM), you need memory barriers — DMB, DSB, ISB on ARM; smp_mb(), smp_rmb(), smp_wmb() in Linux kernel — to ensure ordering across cores.

Q2 Implement a circular buffer (ring buffer) in C for an interrupt-driven UART driver.

#define BUF_SIZE 256  /* Must be power of 2 */

typedef struct {

uint8_t  buf[BUF_SIZE];

uint32_t head;  /* written by producer (ISR) */

uint32_t tail;  /* read by consumer (process context) */

} ring_buf_t;

/* Called from ISR — producer */

static inline int rb_put(ring_buf_t *rb, uint8_t data)

{

uint32_t next = (rb->head + 1) & (BUF_SIZE - 1);

if (next == rb->tail)

return -1;  /* buffer full */

rb->buf[rb->head] = data;

smp_wmb();          /* Ensure data visible before head update */

rb->head = next;

return 0;

}

/* Called from process context — consumer */

static inline int rb_get(ring_buf_t *rb, uint8_t *data)

{

if (rb->head == rb->tail)

return -1;  /* buffer empty */

smp_rmb();          /* Ensure head read before data read */

*data = rb->buf[rb->tail];

rb->tail = (rb->tail + 1) & (BUF_SIZE - 1);

return 0;

}

Key Points

- Power-of-2 size enables fast modulo via bitwise AND: (head + 1) & (SIZE-1)

- smp_wmb() / smp_rmb() provide lock-free single-producer/single-consumer synchronization

- In real kernel UART driver, prefer kfifo (kernel's optimized ring buffer)

- For multi-producer scenarios, use spinlock around rb_put()

Q3 What is the difference between a mutex and a spinlock? When do you use each in a kernel driver?

| Property | Mutex | Spinlock |
| --- | --- | --- |
| Waiting | Sleeps (blocks) | Busy-waits (spins) |
| Context | Process context only | Any context (IRQ safe) |
| Overhead | Higher (context switch) | Lower (no sleep) |
| Hold Time | Can hold long | Must be very short |
| Preemption | Allowed while waiting | Disabled while held |
| Sleep in CS | Allowed (kmalloc GFP_KERNEL) | NEVER sleep |

Usage Rules

- Spinlock: Use in interrupt handlers, softirqs, or critical sections < few microseconds. Use spin_lock_irqsave() when accessed from both process and interrupt context.

- Mutex: Use in process context when critical section may sleep (e.g., copy_to_user, kmalloc(GFP_KERNEL)).

/* Spinlock example — ISR + process context shared data */

static spinlock_t my_lock;

/* In ISR: */

irq_handler_t my_irq(int irq, void *data) {

unsigned long flags;

spin_lock_irqsave(&my_lock, flags);

/* access shared hardware state */

spin_unlock_irqrestore(&my_lock, flags);

return IRQ_HANDLED;

}

ℹ️ Note: Qualcomm-specific: In Qualcomm camera/display drivers, spinlocks protect HW register access from ISR, while mutexes protect higher-level state machines.

SECTION 2: INTERRUPT HANDLING & ARM GICv3

Q4 Describe the ARM GICv3 architecture and how Linux handles interrupts on a multi-core ARM64 SoC.

GICv3 Components

| Component | Description |
| --- | --- |
| Distributor (GICD) | Global. Configures SPI (Shared Peripheral Interrupts, ID 32–1019), enables/disables, sets priority and affinity routing. |
| Redistributor (GICR) | Per-CPU. Manages PPI (Private Peripheral Interrupts, ID 16–31) and SGI (Software Generated Interrupts, ID 0–15). |
| CPU Interface (ICC_*) | Per-CPU system registers accessed via MRS/MSR (no MMIO). ICC_IAR1_EL1 (Acknowledge), ICC_EOIR1_EL1 (End of Interrupt), ICC_PMR_EL1 (Priority Mask), ICC_BPR1_EL1 (Binary Point). |

Interrupt Types

| Type | ID Range | Distribution | Examples |
| --- | --- | --- | --- |
| SGI | 0–15 | Software Generated | IPI (smp_call_function, TLB shootdown, scheduler tick) |
| PPI | 16–31 | Per-CPU Private | ARM generic timer (arch_timer), PMU interrupts |
| SPI | 32–1019 | Shared Peripheral | GPIO, UART, USB, PCIe, Camera CSI |
| LPI | 8192+ | MSI via ITS | PCIe MSI/MSI-X message-signaled interrupts |

Linux Interrupt Flow on ARM64

IRQ Exception

|

v

el1_irq / el0_irq  (arch/arm64/kernel/entry.S)

|   Saves all registers, reads Exception Syndrome

v

handle_arch_irq()  --> GIC driver: gic_handle_irq()

|   Reads ICC_IAR1_EL1 --> get irqnr (interrupt ID)

v

generic_handle_irq(irq) --> irq_desc[irq].handle_irq()

|   For handle_fasteoi_irq: calls action->handler chain

v

action->handler(irq, dev_id)   [hard IRQ handler]

|   Returns IRQ_HANDLED or IRQ_WAKE_THREAD

v

Write ICC_EOIR1_EL1  (EOI - signal priority drop)

Threaded IRQs (IRQF_THREAD)

static irqreturn_t my_hard_irq(int irq, void *data)

{

/* Minimal work: read status reg, clear interrupt */

return IRQ_WAKE_THREAD;  /* Wake the thread */

}

static irqreturn_t my_threaded_irq(int irq, void *data)

{

/* Can sleep here, do heavy processing */

process_data();

return IRQ_HANDLED;

}

/* Registration */

devm_request_threaded_irq(dev, irq,

my_hard_irq,     /* hard IRQ handler */

my_threaded_irq, /* threaded handler - runs as irq/N-name */

IRQF_TRIGGER_RISING, "my-device", dev);

ℹ️ Note: Threaded handler runs in a kernel thread at SCHED_FIFO priority 50. This reduces interrupt latency and allows sleeping in the handler.

IPI (Inter-Processor Interrupts) via SGIs

- SGIs (ID 0–15) are used for software-generated cross-CPU signaling

- smp_call_function(), scheduler_ipi(), TLB shootdown all use IPIs

- gic_raise_softirq() writes to ICC_SGI1R_EL1 to target specific CPUs

/* GICv3: send SGI to CPU 1 */

/* Target list: CPUn = bit n in TargetList field */

/* ICC_SGI1R_EL1 = (TargetList << 0) | (SGI_ID << 24) | (Aff << 16) */

➡ Chain Q: What is the difference between hardirq, softirq, and tasklet?

| Type | Hard IRQ | Softirq | Tasklet |
| --- | --- | --- | --- |
| Context | Interrupt (IRQs disabled) | Interrupt (IRQs enabled) | Interrupt (IRQs enabled) |
| Can sleep | No | No | No |
| Allocation | Static via request_irq | Static (10 types) | Dynamic (tasklet_init) |
| CPU Pinning | Any CPU | Same CPU | Same CPU |
| Runner | CPU handling IRQ | ksoftirqd or IRQ exit | ksoftirqd or IRQ exit |
| Use case | HW IRQ handler | Network RX/TX, timers | Driver deferred work |

ℹ️ Note: For deferred work that can SLEEP, use workqueues (schedule_work(), queue_work()). Workqueue handlers run in kernel thread context.

➡ Chain Q: Explain TLB Shootdown on ARM64 SMP. What instructions are used?

When a kernel modifies a page table entry (unmap, permission change), other CPUs may have stale TLB entries. ARM64 TLB shootdown:

- CPU A modifies PTE, calls flush_tlb_range() or flush_tlb_page()

- ARM64 IS (Inner Shareable) TLBI instructions broadcast automatically to all CPUs in the coherency domain

- No explicit IPI needed on ARM64! (Unlike x86, which requires explicit IPIs)

- DSB ISH ensures TLB invalidation completes before proceeding

TLBI VALE1IS, Xt   /* Invalidate by VA, EL1, Inner Shareable (all CPUs) */

TLBI VMALLE1IS     /* Invalidate all EL1 entries, Inner Shareable */

DSB ISH            /* Data Sync Barrier - wait for TLB invalidation */

ISB                /* Instruction Sync Barrier */

SECTION 3: DEVICE DRIVERS — I2C, SPI, PCIe, USB

Q5 Explain the Linux device driver model — platform_driver, probe(), and Device Tree binding.

Linux Driver Model (LDM) Overview

- Built on kobject / kset / ktype (sysfs backbone)

- Three key abstractions: bus, device, driver

- Bus matches devices to drivers via match() function

- Platform Bus: For SoC peripherals not on discoverable buses (I2C, SPI, UART, GPIO)

- Devices described in Device Tree (DT) or ACPI

Platform Driver Registration

static struct platform_driver my_driver = {

.probe  = my_probe,

.remove = my_remove,

.driver = {

.name           = "my-device",

.of_match_table = my_of_match,

.pm             = &my_pm_ops,

},

};

module_platform_driver(my_driver);  /* Shorthand for init/exit */

Device Tree Binding

my_device@40000000 {

compatible = "vendor,my-device";

reg = <0x40000000 0x1000>;

interrupts = <GIC_SPI 42 IRQ_TYPE_LEVEL_HIGH>;

clocks = <&clk_controller CLK_MY_DEV>;

resets = <&reset_controller RESET_MY_DEV>;

power-domains = <&rpmhpd RPMHPD_CX>;

status = "okay";

};

probe() Function Responsibilities

- platform_get_resource() → get MMIO base address from DT reg property

- devm_ioremap_resource() → map MMIO to kernel virtual address

- platform_get_irq() → get IRQ number (translated from DT)

- devm_request_irq() → register IRQ handler

- devm_clk_get() + clk_prepare_enable() → enable clocks

- devm_reset_control_get() → manage resets

- Register with subsystem (misc_register(), netdev_register(), etc.)

ℹ️ Note: devm_* (device-managed resources) are automatically released on driver unbind or probe() failure, preventing resource leaks.

of_match_table with Multiple Compatibles

static const struct of_device_id my_of_match[] = {

{ .compatible = "vendor,my-device-v1", .data = &v1_data },

{ .compatible = "vendor,my-device-v2", .data = &v2_data },

{ } /* sentinel */

};

MODULE_DEVICE_TABLE(of, my_of_match);

/* In probe(): */

const struct of_device_id *match = of_match_device(my_of_match, dev);

const struct my_variant_data *vdata = match->data;

➡ Chain Q: What is the difference between ioremap, ioremap_nocache, and ioremap_wc?

| Function | Memory Attribute (ARM64) | Use Case |
| --- | --- | --- |
| ioremap() | MT_DEVICE_nGnRnE (non-Gathering, non-Reordering, non-Early Write Ack) | Standard MMIO device registers — strictly ordered |
| ioremap_nocache() | Deprecated alias for ioremap() | Legacy code — same as ioremap() |
| ioremap_wc() | MT_NORMAL_NC or MT_DEVICE_GRE (Write-Combining) | Framebuffers, display memory — write merging allowed |
| ioremap_cache() | MT_NORMAL (cached) | RAM-like regions; rarely used for MMIO |

➡ Chain Q: Explain DMA coherency on ARM. What is the difference between coherent and streaming DMA?

| DMA Type | API | Description |
| --- | --- | --- |
| Coherent DMA | dma_alloc_coherent() | Always coherent between CPU and device. No explicit cache maintenance needed. Expensive (uncached or HW coherent via CCI/CCN). |
| Streaming DMA | dma_map_single(), dma_map_sg() | Uses existing memory. Explicit cache maintenance: TO_DEVICE=flush; FROM_DEVICE=invalidate; BIDIRECTIONAL=both. |
| IOMMU/SMMU | Transparent via dma_map_*() | Maps device DMA addresses (IOVA) to physical addresses. Provides isolation and protection. |

Q7 Walk me through writing an I2C client driver for a sensor on a Qualcomm platform.

/* 1. Define match tables */

static const struct i2c_device_id my_sensor_id[] = {

{ "my-sensor", 0 }, { }

};

MODULE_DEVICE_TABLE(i2c, my_sensor_id);

static const struct of_device_id my_sensor_of_match[] = {

{ .compatible = "vendor,my-sensor" }, { }

};

MODULE_DEVICE_TABLE(of, my_sensor_of_match);

/* 2. probe() */

static int my_sensor_probe(struct i2c_client *client,

const struct i2c_device_id *id)

{

struct my_sensor_data *data;

if (!i2c_check_functionality(client->adapter,

I2C_FUNC_SMBUS_BYTE_DATA))

return -ENODEV;

data = devm_kzalloc(&client->dev, sizeof(*data), GFP_KERNEL);

if (!data) return -ENOMEM;

data->client = client;

i2c_set_clientdata(client, data);

int chip_id = i2c_smbus_read_byte_data(client, REG_CHIP_ID);

if (chip_id != EXPECTED_CHIP_ID) return -ENODEV;

return my_sensor_register(data);

}

/* 3. Driver struct */

static struct i2c_driver my_sensor_driver = {

.driver = { .name = "my-sensor",

.of_match_table = my_sensor_of_match,

.pm = &my_sensor_pm_ops },

.probe  = my_sensor_probe,

.remove = my_sensor_remove,

.id_table = my_sensor_id,

};

module_i2c_driver(my_sensor_driver);

Device Tree for I2C Sensor

&i2c3 {

my_sensor@48 {

compatible = "vendor,my-sensor";

reg = <0x48>;              /* 7-bit I2C address */

interrupt-parent = <&tlmm>;

interrupts = <25 IRQ_TYPE_EDGE_RISING>;

vdd-supply = <&pm8150_l10>; /* Regulator reference */

};

};

➡ Chain Q: What is the difference between i2c_smbus_read_byte_data and i2c_transfer?

- i2c_smbus_*: High-level SMBus protocol wrappers. Simple, limited to SMBus transactions (byte, word, block). Internally calls i2c_transfer.

- i2c_transfer: Low-level, sends raw i2c_msg array. Full control over start/stop/repeated-start conditions. Required for complex transactions (e.g., write register address then read without stop — common for sensors).

/* i2c_transfer: write reg addr + read data (no STOP between) */

struct i2c_msg msgs[2] = {

{ .addr = client->addr, .flags = 0,

.len = 1, .buf = &reg_addr },       /* write */

{ .addr = client->addr, .flags = I2C_M_RD,

.len = 2, .buf = rx_buf }            /* read */

};

i2c_transfer(client->adapter, msgs, 2);

Q8 Explain PCIe enumeration on Linux. What happens when a PCIe device is plugged in?

PCIe Enumeration Flow

- Root Complex (RC) scans bus 0 → reads Vendor/Device ID from config space offset 0x00

- If device found → assigns bus number, BAR (Base Address Register) resources

- pci_scan_bus() → pci_scan_slot() → pci_scan_device() → pci_add_new_bus() for bridges

- pci_assign_resource() → assigns MMIO/IO/MSI resources from parent bridge windows

- pci_bus_add_devices() → pci_device_add() → triggers driver probe()

Qualcomm PCIe Specifics

| Component | Details |
| --- | --- |
| RC Driver | pcie-qcom.c — Qualcomm Root Complex controller driver |
| PHY Driver | phy-qcom-qmp.c — QMP PHY for link training and Gen1/2/3/4 configuration |
| Interrupts | MSI/MSI-X only (no legacy INTx on most Qualcomm platforms) |
| DMA Isolation | ARM SMMU (apps_smmu) used for PCIe DMA isolation; iommus = <&apps_smmu ...> |
| Link Speed | Gen1 (2.5GT/s), Gen2 (5GT/s), Gen3 (8GT/s), Gen4 (16GT/s) per platform |
| Config Space | ECAM (Enhanced Config Access Mechanism) — 256MB window for 256 buses |

➡ Chain Q: What is a BAR (Base Address Register)?

BARs define the device's MMIO/IO regions. A device can have up to 6 BARs (0–5). The OS writes all-1s to a BAR, reads back to determine size (bits that remain 0 = size). Then assigns a physical address. The driver uses pci_ioremap_bar() to map to kernel virtual address.

/* PCIe device driver example */

static int my_pcie_probe(struct pci_dev *pdev,

const struct pci_device_id *id)

{

int err;

err = pci_enable_device(pdev);

if (err) return err;

err = pci_request_regions(pdev, "my-pcie");

if (err) goto err_disable;

void __iomem *bar0 = pci_ioremap_bar(pdev, 0);

pci_set_master(pdev);   /* Enable bus mastering for DMA */

pci_alloc_irq_vectors(pdev, 1, 16, PCI_IRQ_MSI); /* MSI-X */

return 0;

}

Q9 Explain USB enumeration. What is the difference between USB 2.0 and USB 3.x from a driver perspective?

USB Enumeration Sequence

- Device connects → hub detects attach (D+/D- line state change) → resets device

- Host assigns address via SET_ADDRESS control request

- Host reads Device Descriptor (GET_DESCRIPTOR, type=Device)

- Host reads Configuration, Interface, Endpoint, String descriptors

- Host selects configuration (SET_CONFIGURATION)

- usb_match_id() / usb_match_device() matches driver → probe() called

| Aspect | USB 2.0 | USB 3.x (SuperSpeed) |
| --- | --- | --- |
| Max Speed | 480 Mbps (HighSpeed) | 5/10/20 Gbps (SS/SS+) |
| Protocol | Token-based (SOF, IN/OUT tokens; host polls) | Packet-based (no polling; credit-based flow) |
| Bulk Streams | No streams | SuperSpeed Bulk Streams (up to 65535) |
| Linux HCD | EHCI (USB 2.0 only) | xHCI (handles both 2.0 and 3.x) |
| Power Mgmt | VBUS only | VBUS + USB Power Delivery (PD) |
| Qualcomm IP | Synopsys DWC3 (USB2 port) | Synopsys DWC3 (dwc3-qcom.c) |

ℹ️ Note: Qualcomm uses DWC3 (DesignWare USB3) IP core. Driver: drivers/usb/dwc3/. Qualcomm wrapper: dwc3-qcom.c. Supports USB role switching (host/device/OTG) via extcon notifiers.

SECTION 4: QUALCOMM PLATFORM BRING-UP

Q: Explain the Linux Boot Sequence on a Qualcomm MSM/SM Platform.

Power On

|

v

PBL (Primary Boot Loader) -- ROM, embedded in Qualcomm SoC fuses

|  Validates XBL using PKHash chain-of-trust, loads from eMMC/UFS

v

XBL (eXtensible Boot Loader) -- UEFI-based, replaces old SBL

|  Initializes DDR, PMIC, clocks, loads TZ/QSEE, loads ABL

v

ABL (Android Boot Loader) -- Little Kernel based

|  Loads kernel Image/Image.gz, DTB, initramfs from boot partition

|  Reads board ID from SMEM, selects correct DTB

|  Passes bootargs (cmdline) to kernel via DTB chosen node

v

Linux Kernel (Image/Image.gz)

|  Decompress (if needed), setup MMU, cache, stack

v

start_kernel()

|  setup_arch() -- CPU init, MMU, early DT parsing

|  mm_init()    -- Memory subsystem

|  sched_init() -- Scheduler init

|  driver subsystems init (bus, platform, i2c, spi...)

v

kernel_init() / init process (PID 1) --> User Space

| Stage | Component | Key Responsibilities |
| --- | --- | --- |
| Stage 0 | PBL (ROM) | Power-on, authentication of XBL using HW fused root of trust |
| Stage 1 | XBL/SBL | DDR init, PMIC init, clock setup, QSEE/TZ load, ABL load |
| Stage 2 | QSEE (TZ) | Secure world init, SMMU secure config, TA loading capability |
| Stage 3 | ABL (LK) | Fastboot, kernel/DTB load, bootargs, board ID selection |
| Stage 4 | Linux Kernel | Full hardware init, driver probe, subsystem bring-up, init/systemd |

➡ Chain Q: What is the role of DTB in Qualcomm platform bring-up?

The Device Tree Blob (DTB) describes hardware topology to the kernel — peripherals, memory maps, interrupt routing, clock trees, pinmux. On Qualcomm, DTBs are appended to the kernel image or stored separately. The ABL selects the correct DTB based on board ID/SKU read from SMEM (Shared Memory). of_platform_populate() in the kernel instantiates platform devices from DT nodes.

Q10 You are bringing up a new Qualcomm SM8650 platform. The kernel boots but UART console is not working. How do you debug?

Systematic 6-Step Debug Approach

Step 1 — Verify Hardware Path

- Check schematic: Which UART instance? (GENI UART on Qualcomm = UART_SE0 – SE9)

- Verify TX/RX pins with oscilloscope — look for bit-banging on TX line at boot

- Check voltage levels: 1.8V or 3.3V depending on board design

Step 2 — Check Device Tree

/* Required DT settings for Qualcomm GENI UART */

&uart0 {

status = "okay";          /* MUST be okay, not disabled */

pinctrl-names = "default", "sleep";

pinctrl-0 = <&uart0_active>;

pinctrl-1 = <&uart0_sleep>;

/* Verify: reg matches SoC TRM UART instance base address */

/* Verify: clocks = <&gcc GCC_QUPV3_UART0_CLK> is correct */

};

Step 3 — Check Clock/Power Domains

cat /sys/kernel/debug/clk/gcc_qupv3_uart0_clk/clk_enable_count

cat /sys/kernel/debug/regulator/ldo_cx/enable_count

# Verify QUPV3 AHB clock and UART core clock are enabled

Step 4 — Check Pinmux (TLMM)

cat /sys/kernel/debug/pinctrl/pinctrl-handles

cat /sys/kernel/debug/gpio

# GPIO 4/5 should show "mux: qup0_se0_l0 / qup0_se0_l1" for UART

Step 5 — Use earlycon (Pre-Driver Boot Console)

/* Add to bootargs (ABL cmdline): */

earlycon=msm_serial_dm,0x00C1F000 console=ttyMSM0,115200n8

/* earlycon kicks in before platform driver probe() */

/* Helps identify if issue is HW or driver */

Step 6 — JTAG Debug

- Attach Lauterbach TRACE32 via 20-pin debug header

- Load vmlinux with debug symbols

- Set breakpoint at uart_startup() or msm_serial_startup()

- Inspect register state against SoC TRM register map

Q11 What is TLMM (Top Level Mode Multiplexer) on Qualcomm? How do you configure GPIO/pinmux?

TLMM is Qualcomm's pin controller — manages GPIO direction, function mux, drive strength, and pull configuration for all SoC pins. Linux driver: pinctrl-msm.c + SoC-specific file (e.g., pinctrl-sm8650.c).

/* Pin state definition in DT */

uart0_active: uart0-active-state {

pins = "gpio4", "gpio5";

function = "qup0_se0_l0";  /* UART function for these pins */

drive-strength = <2>;       /* 2mA */

bias-disable;               /* No pull */

};

/* GPIO output example */

&tlmm {

gpio_reset: gpio-reset-state {

pins = "gpio10";

function = "gpio";      /* GPIO function (no mux) */

drive-strength = <2>;

output-low;

};

};

/* Runtime GPIO control in driver: */

struct gpio_desc *gpio = devm_gpiod_get(&pdev->dev, "reset", GPIOD_OUT_LOW);

gpiod_set_value_cansleep(gpio, 1);  /* Assert reset */

msleep(10);

gpiod_set_value_cansleep(gpio, 0);  /* Deassert reset */

SECTION 5: TRUSTZONE & QUALCOMM QSEE ARCHITECTURE

Q12 Explain Qualcomm's TrustZone architecture (QSEE). How does a Linux driver communicate with a Trusted Application?

Qualcomm TrustZone (QSEE) Stack

Normal World (HLOS)              Secure World (QSEE)

========================        =======================

User App (QSEECom Client)        Trusted Application (TA)

|                                |

QSEECom library                  QSEE OS (Qualcomm TEE)

|                                |

SCM driver (drivers/firmware/     TrustZone Kernel (EL3)

qcom_scm.c)           |

|                                |

SMC #0 instruction ============> ARM EL3 Secure Monitor

(x0=SMC function ID,            (validates, switches worlds)

x1-x3=arguments)

Communication Flow

- Linux driver calls qcom_scm_call() or qseecom_send_cmd()

- SCM driver issues SMC #0 instruction → traps to EL3

- TrustZone monitor at EL3 validates function ID and arguments

- Monitor saves normal world state, switches to secure world

- QSEE dispatches request to the appropriate Trusted Application

- TA processes request, writes response to shared memory

- Monitor switches back to normal world, returns to EL1

Key Qualcomm SCM Calls

/* Read fuse (OTP) value via TrustZone */

ret = qcom_scm_io_readl(fuse_addr, &fuse_val);

/* Assign memory region permissions between VMs */

ret = qcom_scm_assign_mem(mem_addr, mem_size, &src_vm, &dst_vm, 1);

/* Authenticate and load QSEE trusted application */

ret = qcom_scm_qseecom_app_send(app_id, req_buf, req_size,

rsp_buf, rsp_size);

TrustZone Memory Protection

| Region | Access | Notes |
| --- | --- | --- |
| QSEE Kernel | EL3 only (TZ) | Contains QSEE OS, TAs, secure keys |
| SMEM | Normal + Secure | Shared inter-processor communication |
| Video Secure Memory | Secure World only | DRM/Widevine protected content |
| HLOS DDR | Normal World | Linux kernel + user space memory |

➡ Chain Q: What is SMMU and how does it relate to TrustZone security?

The SMMU (System Memory Management Unit) — Qualcomm's IOMMU — provides DMA isolation. Each peripheral (PCIe, USB, camera) has its own SMMU context bank. TrustZone configures SMMU to prevent non-secure peripherals from accessing secure memory regions. The Linux SMMU driver (arm-smmu.c, arm-smmu-v3.c) manages non-secure context banks. Secure banks are configured exclusively by QSEE via SCM calls.

SECTION 6: YOCTO PROJECT & BUILD SYSTEM

Q13 Explain the Yocto build system. What is a recipe, layer, and bbappend?

| Concept | Type | Description |
| --- | --- | --- |
| Recipe (.bb) | Build instructions | Defines source URL, patches, configure flags, compile, install steps for ONE component |
| Layer (meta-*) | Collection | Group of recipes, configs, classes. Provides modularity. Listed in bblayers.conf. |
| .bbappend | Recipe extension | Extends/overrides an existing recipe without modifying it. Add patches, change configs for your platform. |
| MACHINE | Target HW config | e.g., qcom-armv8. Machine-specific configs in conf/machine/. Sets ARCH, BSP, kernel. |
| DISTRO | Distribution policy | e.g., poky. Controls features, init system (systemd/sysvinit), libc (glibc/musl). |
| IMAGE_INSTALL | Package list | Packages included in final rootfs image. |
| bitbake | Build engine | Parses recipes, resolves dependencies (DEPENDS/RDEPENDS), executes tasks (do_fetch, do_compile, do_install) |

Recipe Structure Example

DESCRIPTION = "My custom kernel module for Qualcomm platform"

LICENSE = "GPL-2.0-only"

LIC_FILES_CHKSUM = "file://COPYING;md5=..."

SRC_URI = "git://github.com/vendor/my-module.git;branch=main \

file://0001-fix-build-warning.patch"

SRCREV = "a1b2c3d4e5f6..."

inherit module       # For kernel modules

KERNEL_MODULE_AUTOLOAD += "my_module"

KERNEL_MODULE_PROBECONF += "my_module"

Common bitbake Commands

| Command | Description |
| --- | --- |
| bitbake core-image-minimal | Build a minimal rootfs image |
| bitbake -c devshell <recipe> | Open interactive shell in recipe build environment |
| bitbake -e <recipe> ¦ grep ^SRC_URI | Inspect resolved variable values for a recipe |
| bitbake-layers show-layers | List all active layers and priorities |
| bitbake -c cleanall <recipe> | Clean recipe build artifacts and sstate cache |
| bitbake -g <recipe> | Generate dependency graph (task-depends.dot) |
| devtool modify <recipe> | Check out source for development/patching |

ℹ️ Note: Qualcomm provides meta-qcom BSP layer with recipes for MSM/SM platform kernel, proprietary firmware blobs, QSEE, and peripheral firmware. Access requires Qualcomm CodeLinaro membership.

SECTION 7: DEBUGGING TOOLS — JTAG, ftrace, perf, QDSS

Q14 How do you use JTAG for kernel debugging on a Qualcomm platform?

JTAG Debug Setup

- Hardware: Lauterbach TRACE32 or ARM DS-5 + 20-pin JTAG/SWD connector on board

- Qualcomm: QDSS (Qualcomm Debug Subsystem) provides unified debug interface

- Ensure kernel built with CONFIG_DEBUG_INFO=y, CONFIG_FRAME_POINTER=y

JTAG Workflow

- Connect JTAG probe to target board debug header (JTAG/SWD)

- Load vmlinux (with DWARF debug symbols) into TRACE32/GDB

- Set hardware breakpoints: b platform_driver_probe

- Inspect CPU registers, kernel stacks, memory regions

- For post-crash analysis: load vmcore + vmlinux into crash utility

QDSS (Qualcomm Debug Subsystem)

| Component | Description |
| --- | --- |
| ETM | Embedded Trace Macrocell — instruction-level tracing. Captures PC history with timestamps. Enables function call tracing without breakpoints. |
| STM | System Trace Macrocell — software-inserted trace points. Lower overhead than ETM. Used for printk-like tracing in production. |
| Funnel | Aggregates trace streams from multiple cores into single pipeline. |
| TMC/ETB | Trace Memory Controller / Embedded Trace Buffer — stores trace data in on-chip SRAM. |
| TPIU | Trace Port Interface Unit — outputs trace to external analyzer via trace port. |
| Linux Driver | drivers/hwtracing/coresight/ — CoreSight framework (coresight-etm4x.c, coresight-stm.c) |

Oscilloscope Use Cases in Bring-Up

- Verify I2C/SPI signal integrity — rise time, voltage levels, noise

- Measure interrupt latency — GPIO toggle at ISR entry/exit vs. hardware event

- Debug clock issues — verify PLL lock, output frequency accuracy

- Check UART baud rate accuracy — measure bit timing

- I2C/SPI protocol decode — oscilloscope protocol analyzers validate transactions

Q15 What is ftrace and how do you use it for kernel debugging?

ftrace — Linux Kernel Tracing Framework

# Enable function tracing

cd /sys/kernel/debug/tracing

echo function > current_tracer

echo 1 > tracing_on

cat trace

echo 0 > tracing_on

# Trace a specific driver function

echo my_driver_irq_handler > set_ftrace_filter

echo function_graph > current_tracer

echo 1 > tracing_on && sleep 1 && cat trace

# Trace events (preferred over function tracing for production)

echo 1 > events/irq/irq_handler_entry/enable

echo 1 > events/sched/sched_switch/enable

cat trace_pipe   # Stream live events

trace-cmd and perf

# trace-cmd (user-space wrapper for ftrace)

trace-cmd record -p function_graph -g my_driver_probe

trace-cmd report | head -100

# perf for performance profiling

perf stat -e cache-misses,cache-references,cycles,instructions ./app

perf record -g -p <pid>    # Record with call graph

perf report                # Analyze hotspots

perf top                   # Live CPU profiling

# Qualcomm: Decode kernel crash backtrace

scripts/decode_stacktrace.sh vmlinux < oops.txt

addr2line -e vmlinux -i 0xffffffc0081234ab

Kernel Debug Config Options

| Config Option | Purpose |
| --- | --- |
| CONFIG_DEBUG_INFO | Include DWARF debug info in vmlinux (needed for JTAG/crash) |
| CONFIG_KASAN | Kernel Address Sanitizer — detects use-after-free, out-of-bounds |
| CONFIG_UBSAN | Undefined Behavior Sanitizer — integer overflow, misaligned access |
| CONFIG_LOCKDEP | Lock dependency validator — detects deadlocks at runtime |
| CONFIG_PROVE_LOCKING | Enables full lock correctness checking |
| CONFIG_FTRACE | Enables the ftrace kernel tracing infrastructure |
| CONFIG_DYNAMIC_DEBUG | Enables pr_debug() / dev_dbg() at runtime via debugfs |
| CONFIG_KGDB | Kernel GDB stub — debug kernel over serial/network |

SECTION 8: POWER MANAGEMENT — RUNTIME PM & RPMH

Q16 Explain Linux runtime PM (rpm). How does a Qualcomm peripheral driver implement it?

Runtime PM Concept

Runtime PM (RPM) allows individual devices to be suspended/resumed dynamically based on usage, independent of system suspend (S3). It tracks usage count — when count reaches 0 and autosuspend delay expires, the device is suspended.

Runtime PM API in a Driver

/* In probe() — initialize runtime PM */

pm_runtime_enable(&pdev->dev);

pm_runtime_set_active(&pdev->dev);   /* Device is initially active */

pm_runtime_set_autosuspend_delay(&pdev->dev, 50); /* 50ms delay */

pm_runtime_use_autosuspend(&pdev->dev);

/* Before accessing hardware (increments usage count) */

ret = pm_runtime_get_sync(&pdev->dev);

if (ret < 0) {

pm_runtime_put_noidle(&pdev->dev);

return ret;

}

/* ... do hardware access ... */

/* After done (decrements count; autosuspend after delay) */

pm_runtime_put_autosuspend(&pdev->dev);

/* Runtime PM callbacks in dev_pm_ops */

static int my_runtime_suspend(struct device *dev)

{

clk_disable_unprepare(priv->clk);

regulator_disable(priv->vdd);

return 0;

}

static int my_runtime_resume(struct device *dev)

{

regulator_enable(priv->vdd);

return clk_prepare_enable(priv->clk);

}

static const struct dev_pm_ops my_pm_ops = {

SET_RUNTIME_PM_OPS(my_runtime_suspend, my_runtime_resume, NULL)

SET_SYSTEM_SLEEP_PM_OPS(my_system_suspend, my_system_resume)

};

Qualcomm RPMH (Resource Power Manager Hardened)

Qualcomm platforms use RPMH — a dedicated ARC (Always-on Resource Controller) processor managing shared resources: voltage rails, clocks, buses. Linux communicates asynchronously via TCS (Triggered Command Set) writes.

RPMH Architecture:

Linux Kernel                   RPMH (ARC Processor)

============                   ======================

rpmhpd_set_performance_state()

|

v  (vote via cmd-db)

RPMH RSC (Resource State Coordinator)  -->  ARC

|   TCS write: VRM (voltage), BCM (bus)   |

|                                          v

SLEEP set TCS  -----> sent on CPU idle    PMIC / Clock / Bus

WAKE set TCS   -----> sent on CPU wakeup  configuration

ACTIVE set TCS -----> sent immediately

| RPMH Component | Function | Linux Driver |
| --- | --- | --- |
| RSC | Resource State Coordinator — arbitrates votes | drivers/soc/qcom/rpmh-rsc.c |
| VRM | Voltage Rail Manager — controls PMICs | drivers/regulator/qcom-rpmh-regulator.c |
| BCM | Bus Clock Manager — manages bus bandwidth | drivers/interconnect/qcom/ |
| CMD-DB | Command Database — resource address lookup | drivers/soc/qcom/cmd-db.c |
| RPMHPD | Power domain driver using RPMH votes | drivers/pmdomain/qcom/rpmhpd.c |

SECTION 9: QUALCOMM-SPECIFIC TOPICS — SMEM & QMI

Q17 What is SMEM (Shared Memory) on Qualcomm platforms?

SMEM is a fixed shared memory region used for inter-processor communication (IPC) between the Application Processor (APSS/Linux) and other subsystems: MPSS (modem), ADSP (audio DSP), CDSP (compute DSP), WCSS (Wi-Fi/BT).

SMEM Architecture

Physical DDR

+-----------------------+ 0x86200000 (example Qualcomm address)

|   SMEM Region         |   Size: ~2MB, shared across all processors

|  +-----------------+  |

|  | SMEM Header     |  |   Table of items (smem_item_table)

|  +-----------------+  |

|  | Item 0: BoardInfo|  |   Each item: ID + size + data

|  | Item 1: BootInfo |  |

|  | Item 2: ProcComm |  |

|  | Item N: ...      |  |

|  +-----------------+  |

+-----------------------+

SMEM Usage in Linux

/* Linux driver: drivers/soc/qcom/smem.c */

/* Device Tree: */

memory@86000000 {

compatible = "qcom,smem";

reg = <0x86000000 0x200000>;  /* Physical addr, size */

no-map;                       /* Not mapped in kernel */

hwlocks = <&tcsr_mutex 3>;    /* TCSR mutex for access protection */

};

/* Reading SMEM item from driver: */

#include <linux/soc/qcom/smem.h>

size_t size;

void *ptr = qcom_smem_get(QCOM_SMEM_HOST_ANY,

SMEM_BOARD_INFO, &size);

if (!IS_ERR(ptr)) {

struct smem_board_info *board = ptr;

dev_info(dev, "Board ID: %d, HW version: %d",

board->board_id, board->hw_ver);

}

Common SMEM Item IDs

| SMEM Item ID | Content | Used By |
| --- | --- | --- |
| SMEM_BOARD_INFO | Board ID, HW revision, SKU | ABL (DTB selection), kernel drivers |
| SMEM_BOOT_INFO | Boot status, warm boot flag | Bootloader, kernel init |
| SMEM_VERSION_INFO | Software versions per subsystem | Diagnostic/bug reports |
| SMEM_MODEM_REASON | Modem crash reason/dump info | SSR (Subsystem Restart) driver |
| SMEM_POWER_ON_STATUS | Power-on reason (cold/warm) | PMIC driver, diagnostics |

Q18 What is QMI (Qualcomm MSM Interface) and how does it work?

QMI is Qualcomm's IPC (Inter-Process Communication) protocol used for structured message-based communication between HLOS (Linux) and remote subsystems (modem/MPSS, ADSP, WCSS).

QMI Architecture

Linux Application Processor (HLOS)      Remote Subsystem (MPSS/ADSP)

==============================          ===========================

QMI Client (e.g., netdev driver)        QMI Service (e.g., modem)

|                                       |

QMI Helper (lib/qmi_encdec.c)              QMI Service Handler

| (encode/decode TLV messages)          |

QRTR Socket (AF_QIPCRTR)  <============>  IPC Router (QRTR)

|   (Qualcomm Router - socket IPC)       |

Shared Memory Transport                  Shared Memory Transport

(via SMEM or SMP2P)                      (via SMEM or SMP2P)

| Component | Description |
| --- | --- |
| QMI IDL | Interface Definition Language — defines message structures (similar to Protobuf). Auto-generates encode/decode code. |
| QRTR | Qualcomm Router — socket-based IPC (AF_QIPCRTR). Multiplexes multiple QMI services over shared transport. drivers/net/qrtr/ |
| SMP2P | Shared Memory Point-to-Point — lightweight signaling mechanism. Used for subsystem notifications. |
| TLV | Type-Length-Value encoding format used by QMI messages for extensibility and backward compatibility. |

QMI Flow Example

/* Linux QMI client (e.g., Wi-Fi firmware control) */

#include <linux/soc/qcom/qmi.h>

struct qmi_handle qmi;

/* Initialize QMI handle */

qmi_handle_init(&qmi, MAX_MSG_SIZE, &ops, NULL);

qmi_add_lookup(&qmi, service_id, version, instance);

/* Send synchronous QMI request */

ret = qmi_send_request(&qmi, sq, &txn,

msg_id, msg_size,

req_ei, &req_struct);

/* Wait for response */

ret = qmi_txn_wait(&txn, msecs_to_jiffies(QMI_TIMEOUT_MS));

ℹ️ Note: QMI is used by: Wi-Fi (WCNSS/WCN6xxx), Modem data (rmnet), Sensors (ADSP), Thermal (CDSP), Subsystem Restart (SSR). Understanding QMI is essential for Qualcomm platform debugging.

Q6 What is a kernel oops vs. kernel panic? How do you debug them?

| Type | Kernel Oops | Kernel Panic |
| --- | --- | --- |
| Severity | Non-fatal (by default) | Fatal — system halts |
| Cause | NULL ptr deref, bad memory access, BUG() | panic(), double fault, oops in critical path |
| System State | Continues (but often unstable) | Halts, optionally reboots |
| Config | panic_on_oops=1 converts to panic | panic_timeout=N for auto-reboot |

Debugging Steps

- Decode backtrace: scripts/decode_stacktrace.sh vmlinux < oops.txt

- addr2line: addr2line -e vmlinux -i 0xffffffc0081234ab

- objdump: objdump -d vmlinux | grep -A20 <fault_addr>

- KASAN: Kernel Address Sanitizer catches use-after-free and OOB

- KGDB/JTAG: Attach GDB via JTAG for live debugging

- Qualcomm ramdump: Captures full DDR on panic; analyze with crash utility

➡ Chain Q: What does the oops output tell you?

- PC (Program Counter): Exact instruction that faulted

- LR (Link Register) / x30: Caller function

- SP: Stack pointer at fault time

- ESR_EL1: Exception syndrome — fault type (translation, permission, alignment)

- FAR_EL1: The virtual address that caused the fault

- Call Stack: Full backtrace from fault to entry point

SECTION 10: CODING QUESTIONS

Q19 Reverse the bits of a 32-bit integer. (Common Qualcomm coding question)

Approach 1: Simple Loop — O(32)

uint32_t reverse_bits(uint32_t n)

{

uint32_t result = 0;

int i;

for (i = 0; i < 32; i++) {

result = (result << 1) | (n & 1);  /* LSB of n -> MSB of result */

n >>= 1;

}

return result;

}

Approach 2: Optimized Bit-Swap — O(log 32) = O(5)

uint32_t reverse_bits_fast(uint32_t n)

{

/* Swap 16-bit halves */

n = ((n & 0xFFFF0000) >> 16) | ((n & 0x0000FFFF) << 16);

/* Swap 8-bit quarters */

n = ((n & 0xFF00FF00) >>  8) | ((n & 0x00FF00FF) <<  8);

/* Swap 4-bit nibbles */

n = ((n & 0xF0F0F0F0) >>  4) | ((n & 0x0F0F0F0F) <<  4);

/* Swap 2-bit pairs */

n = ((n & 0xCCCCCCCC) >>  2) | ((n & 0x33333333) <<  2);

/* Swap individual bits */

n = ((n & 0xAAAAAAAA) >>  1) | ((n & 0x55555555) <<  1);

return n;

}

Example: reverse_bits(0x80000000)

Input:  0x80000000 = 1000 0000 0000 0000 0000 0000 0000 0000

Output: 0x00000001 = 0000 0000 0000 0000 0000 0000 0000 0001

Q20 Detect a cycle in a linked list. (Classic DS question asked at Qualcomm)

Floyd's Cycle Detection — Tortoise and Hare

struct ListNode {

int val;

struct ListNode *next;

};

/* Detect cycle — O(n) time, O(1) space */

bool has_cycle(struct ListNode *head)

{

struct ListNode *slow = head, *fast = head;

while (fast && fast->next) {

slow = slow->next;           /* moves 1 step */

fast = fast->next->next;    /* moves 2 steps */

if (slow == fast)            /* they meet? cycle! */

return true;

}

return false;

}

/* Find cycle start node */

struct ListNode *find_cycle_start(struct ListNode *head)

{

struct ListNode *slow = head, *fast = head;

while (fast && fast->next) {

slow = slow->next;

fast = fast->next->next;

if (slow == fast) {

/* Reset slow to head, keep fast at meeting point */

slow = head;

while (slow != fast) {

slow = slow->next;

fast = fast->next;

}

return slow;  /* cycle start */

}

}

return NULL;

}

Q21 Count the number of set bits (1s) in a 32-bit integer.

/* Brian Kernighan's Algorithm — O(number of set bits) */

int count_bits(uint32_t n)

{

int count = 0;

while (n) {

n &= (n - 1);  /* Clears the lowest set bit */

count++;

}

return count;

}

/* GCC built-in (preferred in practice): */

int count = __builtin_popcount(n);

/* Key insight: n & (n-1) always clears the rightmost set bit */

/* Example: n=0b1100 -> n-1=0b1011 -> n&(n-1)=0b1000 */

SECTION 11: QUALCOMM INTERVIEW PROCESS & PRIORITY TOPICS

Interview Process at Qualcomm — Staff Engineer Level

| Round | Format | Duration | Focus Areas |
| --- | --- | --- | --- |
| HR Screen | Phone | 30 min | Background, motivation, compensation expectations |
| Tech Screen | Phone/Video | 60 min | C/C++ fundamentals, OS basics, one coding problem |
| Onsite R1 | In-person | 60 min | Deep dive on most complex project you have delivered |
| Onsite R2 | In-person | 60 min | Linux kernel internals + device driver development |
| Onsite R3 | In-person | 60 min | ARM architecture + Qualcomm platform bring-up |
| Onsite R4 | In-person | 60 min | Coding: DSA — linked list, trees, bit manipulation |
| Onsite R5 | In-person | 60 min | System design: e.g., design power management for new SoC |
| HM Round | In-person | 45 min | Leadership, cross-team collaboration, roadmap vision |

ℹ️ Note: Qualcomm interviews are known for pivoting mid-question. Example: starting with "explain SMMU" and pivoting to "show me how you would debug an SMMU fault in a PCIe driver." Prepare to go 3 levels deep on any topic.

Priority Topics to Brush Up (Ordered by Impact)

| # | Topic | Why Critical | Key Concepts |
| --- | --- | --- | --- |
| 1 | Qualcomm GENI | Unique to Qualcomm — asked in 90% of interviews | Unified SE (Serial Engine) for I2C/SPI/UART, GSI DMA, FIFO vs DMA mode |
| 2 | RPMH / BCM | Platform-specific PM — separates Qualcomm experts | TCS (SLEEP/WAKE/ACTIVE sets), VRM, BCM, cmd-db, rpmhpd |
| 3 | Camera Bring-up | Major Qualcomm revenue — deep V4L2 knowledge needed | V4L2 subdev framework, CSI-2 lanes, CAMSS pipeline, ISP |
| 4 | QSEECOM / SCM | TrustZone is core to all Qualcomm products | qcom_scm.c, SMC IDs, QSEECOM ioctls, TA lifecycle |
| 5 | Subsystem Restart | Modem/ADSP crash recovery is everyday Qualcomm work | remoteproc framework, SSR notifiers, crash dump, PIL |
| 6 | IOMMU / SMMU | DMA security is increasingly important | Context banks, stream IDs, iommu_domain, SMMU faults |
| 7 | Concurrency Primitives | Always asked at Staff level for correctness | RCU, seqlock, per-CPU variables, lockless data structures |

System Call Path on ARM64 — Quick Reference

1. User space: syscall number in x8, args in x0-x5, then: SVC #0

2. CPU: synchronous exception to EL1 -> VBAR_EL1 + 0x400

3. el0_sync (entry.S): save pt_regs, read ESR_EL1 (EC=0x15=SVC)

4. el0_svc() -> do_el0_svc(): index into sys_call_table[x8]

5. Execute syscall handler (e.g., sys_read, sys_write, sys_open)

6. Return: restore pt_regs, ERET to EL0

Qualcomm SCM (Secure Channel Manager):

EL1 (Linux) -> SMC #0 -> EL3 (TrustZone) -> Secure World

x0 = function ID (owner | service | function_num)

x1-x5 = arguments

Linux Boot Sequence on Qualcomm — Kernel Init

start_kernel() {

setup_arch();         /* CPU, MMU, early DT, BSS clear */

setup_log_buf();

mm_init();            /* Buddy allocator, slab, vmalloc */

sched_init();

rcu_init();

early_irq_init();

init_IRQ();           /* GIC init */

time_init();          /* ARM generic timer */

softirq_init();

driver_init();        /* Core bus types: platform, i2c, spi */

of_core_init();       /* Device Tree node population */

rest_init();          /* Create kthreadd, call kernel_init() */

}

QUICK REFERENCE: COMMON QUALCOMM INTERVIEW TIPS

| DO | AVOID |
| --- | --- |
| Always mention Qualcomm-specific components (GENI, RPMH, TLMM, QSEE) when relevant | Generic answers that ignore Qualcomm platform specifics |
| Draw ASCII diagrams or block diagrams to explain architectures | Vague hand-waving without concrete register/API references |
| Reference actual Linux kernel source files/functions | Saying "the driver handles it" without explaining how |
| Volunteer debugging approach: hardware first, then DT, then driver | Jumping straight to code without verifying hardware setup |
| Show depth: mention KPTI/Spectre mitigations, RCU, lockless design | Staying at surface level for senior/staff questions |
| Admit when uncertain, explain your reasoning process | Bluffing or guessing specific register values without confidence |

—  End of Part 2  —  Sandeep Kumar • Qualcomm Staff Engineer Interview Guide