Document 3 of 3

QUALCOMM BOOTLOADER INTERVIEW PREPARATION

Hibernation Implementation & Boot Time Optimization

ABL • UEFI • Qualcomm Platforms • ARM64 Assembly

| TOPIC | COVERAGE |
| --- | --- |
| Part 1 | Complete Hibernation Implementation in ABL - What/Why/How, Architecture, Image Format, Security Verification, Memory Restoration, CPU State Restoration with ARM64 Assembly Trampoline, Image Invalidation, Key Challenges, Decision Flow, All Cross-Questions |
| Part 2 | Complete Boot Time Optimization Guide - Measurement Techniques (4 methods with code), Stage-by-Stage Optimizations (PBL, XBL with DDR fast boot, ABL with AVB/display, Kernel with compression comparison, Android Init), Summary Table, Real-World Before/After, All Cross-Questions |

PART 1: HIBERNATION IMPLEMENTATION IN ABL (UEFI ABL)

CONTEXT: This covers bootloader-based hibernation on Qualcomm platforms, specifically for automotive use cases (IVI - In-Vehicle Infotainment) where fast resume time is critical. Moves image restoration from a restore-kernel into ABL itself, dramatically reducing time to first picture on screen.

1.1  What Is Hibernation in the Bootloader Context?

Hibernation (also called Suspend-to-Disk or ACPI S4) is a power management state where the entire system memory (RAM) contents are saved to persistent storage (eMMC/UFS) before the system powers off completely. On resume, instead of doing a full cold boot, the bootloader restores the saved memory image and jumps back to the kernel's resume entry point - bypassing the entire Linux init sequence and restoring the system to its exact pre-hibernation state.

Why Implement Hibernation in ABL?

| Approach | Resume Path | Time to Display |
| --- | --- | --- |
| Traditional Linux Hibernation | PBL → XBL → ABL → Kernel → Restore Kernel → Resume | ~3-5 seconds |
| ABL-Based Hibernation | PBL → XBL → ABL → Restore Image → Resume Kernel | ~1-2 seconds |

Key Motivation

- Eliminates restore kernel overhead: Standard approach needs a full kernel boot cycle (~1-2s) just to load the hibernation image.

- Automotive requirement: IVI rear-view camera must display within 2 seconds of ignition - regulatory requirement.

- ABL directly restores memory: ABL reads the image from storage into DDR and jumps to kernel's resume entry point without booting a restore kernel.

1.2  Architecture Overview

Hibernation Entry (Suspend-to-Disk)

Running Linux Kernel

|

| echo disk > /sys/power/state

v

Kernel Hibernation Path:

1. Freeze all processes

2. Create memory snapshot

3. Write snapshot to swap/hibernate partition on eMMC/UFS

4. Power down (S4/S5)

|

v

[POWER OFF]

Hibernation Resume (ABL-Based)

[POWER ON]

|

v

PBL  (Normal cold boot path)

|

v

XBL/SBL  (DDR init, hardware setup)

|

v

+---------------------------------------------+

|    ABL (UEFI ABL) -- RESUME PATH             |

|                                              |

|  1. Detect hibernation image on disk         |

|  2. Validate image integrity & signature     |

|  3. Load image directly into DDR             |

|  4. Restore CPU/peripheral state             |

|  5. Jump to kernel resume entry point        |

+---------------------------------------------+

|

v

Kernel Resume Path:

1. Restore device states

2. Thaw processes

3. System fully resumed (display ON instantly)

1.3  Detailed Implementation - All 6 Sections

Section 1: Hibernation Image Detection

When ABL starts, before proceeding with normal kernel boot, it checks for a valid hibernation image:

// In ABL's BootLinux.c or equivalent

EFI_STATUS BootLinuxFromHibernation(VOID)

{

EFI_STATUS      Status;

HIBERNATE_HEADER HibHeader;

// Step 1: Check if hibernate partition exists

Status = FindPartition("hibernate", &HibPartHandle);

if (EFI_ERROR(Status)) {

DEBUG((EFI_D_INFO, "[HIB] No hibernate partition, normal boot\n"));

return EFI_NOT_FOUND;

}

// Step 2: Read hibernate image header

Status = ReadFromPartition(HibPartHandle, 0,

sizeof(HibHeader), &HibHeader);

if (EFI_ERROR(Status)) {

return EFI_DEVICE_ERROR;

}

// Step 3: Validate magic number

if (HibHeader.Magic != HIBERNATE_MAGIC) {

DEBUG((EFI_D_INFO, "[HIB] No valid image, normal boot\n"));

return EFI_NOT_FOUND;

}

// Step 4: Check image version compatibility

if (HibHeader.Version != HIBERNATE_VERSION_CURRENT) {

DEBUG((EFI_D_WARN, "[HIB] Version mismatch, invalidating\n"));

InvalidateHibernateImage(HibPartHandle);

return EFI_INCOMPATIBLE_VERSION;

}

DEBUG((EFI_D_INFO, "[HIB] Valid image found! Resuming...\n"));

return ResumeFromHibernation(&HibHeader, HibPartHandle);

}

Section 2: Hibernate Image Format Struct

The hibernation image stored on disk has a specific format that tracks all metadata needed for safe restoration:

typedef struct {

UINT32  Magic;              // 0x53574150 ("SWAP") or custom magic

UINT32  Version;            // Image format version

UINT64  ImageSize;          // Total size of memory snapshot

UINT64  PageCount;          // Number of memory pages saved

UINT64  KernelResumeAddr;   // Kernel resume entry point (virtual addr)

UINT64  KernelResumePhys;   // Physical address for resume trampoline

UINT64  CpuStateOffset;     // Offset to saved CPU register state

UINT64  PageTableBase;      // Saved TTBR0/TTBR1 value

UINT32  CrcChecksum;        // CRC32 of the entire image

UINT8   Signature[256];     // RSA signature for secure boot verification

UINT32  CompressionType;    // 0=none, 1=LZO, 2=LZ4

UINT64  CompressedSize;     // Size after compression

// Page bitmap follows - indicates which pages are saved

// Actual page data follows the bitmap

} HIBERNATE_HEADER;

Section 3: Image Integrity and Security Verification

This is critical for secure boot compliance. ABL must verify the hibernation image has not been tampered with, as the image contains the entire kernel memory state including credentials, keys, and security contexts.

EFI_STATUS VerifyHibernateImage(HIBERNATE_HEADER *Header,

VOID *ImageData)

{

EFI_STATUS Status;

// Step 1: Verify CRC integrity

UINT32 CalculatedCrc = CalculateCrc32(ImageData,

Header->ImageSize);

if (CalculatedCrc != Header->CrcChecksum) {

DEBUG((EFI_D_ERROR, "[HIB] CRC mismatch! Image corrupted\n"));

return EFI_CRC_ERROR;

}

// Step 2: Verify cryptographic signature (secure boot chain)

// The hibernate image must be signed with the same key chain

// used for boot.img verification

Status = VerifyImageSignature(

Header->Signature,

sizeof(Header->Signature),

ImageData,

Header->ImageSize,

GetOemPublicKey()

);

if (EFI_ERROR(Status)) {

DEBUG((EFI_D_ERROR, "[HIB] Signature verification FAILED!\n"));

return EFI_SECURITY_VIOLATION;

}

// Step 3: Verify kernel version matches

// Prevent resuming into a different kernel version

Status = ValidateKernelVersion(Header->KernelVersion);

if (EFI_ERROR(Status)) {

DEBUG((EFI_D_WARN, "[HIB] Kernel version mismatch, invalidating\n"));

return EFI_INCOMPATIBLE_VERSION;

}

return EFI_SUCCESS;

}

Why Security Verification Is Critical

- Without verification: An attacker could craft a malicious hibernation image that, when restored, gives them kernel-level access.

- Image contains: Entire kernel memory state including credentials, encryption keys, and security contexts.

- Secure boot chain: Must extend to the hibernation image using the same OEM key chain as boot.img.

- Rollback protection: A separate rollback index for the hibernate image prevents downgrade attacks.

Section 4: Memory Image Restoration

This is the core of ABL-based hibernation. The saved memory pages are loaded directly from storage back into DDR, with optional LZ4 decompression:

EFI_STATUS RestoreMemoryImage(HIBERNATE_HEADER *Header,

EFI_HANDLE PartHandle)

{

EFI_STATUS Status;

UINT64     Offset = sizeof(HIBERNATE_HEADER);

UINT64     PagesRestored = 0;

VOID      *DecompBuffer = NULL;

// Step 1: Allocate decompression buffer if compressed

if (Header->CompressionType != COMPRESS_NONE) {

DecompBuffer = AllocatePages(DECOMP_BUFFER_PAGES);

if (!DecompBuffer) return EFI_OUT_OF_RESOURCES;

}

// Step 2: Read page bitmap (which pages were saved)

UINT64 BitmapSize = (Header->PageCount + 7) / 8;

UINT8 *PageBitmap = AllocatePool(BitmapSize);

Status = ReadFromPartition(PartHandle, Offset,

BitmapSize, PageBitmap);

Offset += BitmapSize;

// Step 3: Restore pages one by one (or in chunks)

for (UINT64 PageIdx = 0;

PageIdx < Header->PageCount; PageIdx++) {

if (!IsBitSet(PageBitmap, PageIdx)) continue;

UINT64 PhysAddr = PageIdx * EFI_PAGE_SIZE;  // 4KB pages

if (Header->CompressionType == COMPRESS_LZ4) {

// Read compressed chunk

UINT32 CompChunkSize;

ReadFromPartition(PartHandle, Offset,

4, &CompChunkSize);

Offset += 4;

ReadFromPartition(PartHandle, Offset,

CompChunkSize, DecompBuffer);

Offset += CompChunkSize;

// Decompress directly to target physical address

LZ4_decompress_safe(DecompBuffer, (VOID*)PhysAddr,

CompChunkSize, EFI_PAGE_SIZE);

} else {

// Uncompressed - read directly to physical address

ReadFromPartition(PartHandle, Offset,

EFI_PAGE_SIZE, (VOID*)PhysAddr);

Offset += EFI_PAGE_SIZE;

}

PagesRestored++;

if (PagesRestored % 10000 == 0) {

DEBUG((EFI_D_INFO, "[HIB] Restored %lu/%lu pages\n",

PagesRestored, Header->PageCount));

}

}

FreePool(PageBitmap);

if (DecompBuffer)

FreePages(DecompBuffer, DECOMP_BUFFER_PAGES);

return EFI_SUCCESS;

}

Section 5: CPU State Restoration and Kernel Jump

After memory is restored, ABL must carefully set up CPU state and jump to the kernel's resume entry point. This requires precise control over ARM64 system registers:

EFI_STATUS JumpToKernelResume(HIBERNATE_HEADER *Header)

{

CPU_STATE *SavedState;

// Step 1: Read saved CPU state from image

SavedState = (CPU_STATE*)(Header + Header->CpuStateOffset);

// Step 2: Invalidate all caches

// Critical: stale cache entries cause data corruption

InvalidateDataCache();

InvalidateInstructionCache();

// Step 3: Invalidate TLB

// Old TLB entries from ABL's own page tables must be flushed

InvalidateTlb();

// Step 4: Disable MMU

DisableMmu();

// Step 5-7: Assembly trampoline for precise register control

HibernateResumeTrampoline(

Header->KernelResumePhys,  // Resume entry point

SavedState                  // Pointer to saved CPU state

);

CpuDeadLoop();  // Should never reach here

return EFI_DEVICE_ERROR;

}

ARM64 Assembly Trampoline (hibernate_resume.S)

The assembly trampoline restores all ARM64 system registers and performs the final jump to kernel resume. Running with MMU disabled to avoid page table conflicts between ABL and kernel:

// hibernate_resume.S

.global HibernateResumeTrampoline

HibernateResumeTrampoline:

// x0 = kernel resume physical address

// x1 = pointer to saved CPU state

// 1. Disable interrupts

msr     daifset, #0xf

// 2. Clean and invalidate all data caches

bl      flush_dcache_all

// 3. Invalidate instruction cache

ic      iallu

dsb     sy

isb

// 4. Invalidate TLB

tlbi    vmalle1

dsb     sy

isb

// 5. Disable MMU, data cache, instruction cache

mrs     x2, sctlr_el1

bic     x2, x2, #(1 << 0)   // Clear M bit (MMU)

bic     x2, x2, #(1 << 2)   // Clear C bit (Data cache)

bic     x2, x2, #(1 << 12)  // Clear I bit (Instr cache)

msr     sctlr_el1, x2

isb

// Now running with MMU off, physical addressing

// 6. Load saved ARM64 system register values

ldr     x2, [x1, #CPU_STATE_TTBR0]   // Page table base 0

ldr     x3, [x1, #CPU_STATE_TTBR1]   // Page table base 1

ldr     x4, [x1, #CPU_STATE_TCR]     // Translation control

ldr     x5, [x1, #CPU_STATE_MAIR]    // Memory attributes

ldr     x6, [x1, #CPU_STATE_SCTLR]   // System control

ldr     x7, [x1, #CPU_STATE_VBAR]    // Vector base addr

ldr     x8, [x1, #CPU_STATE_SP]      // Stack pointer

// 7. Restore page table registers

msr     ttbr0_el1, x2

msr     ttbr1_el1, x3

msr     tcr_el1, x4

msr     mair_el1, x5

msr     vbar_el1, x7

msr     sp_el1, x8

isb

// 8. Re-enable MMU with kernel's page tables

msr     sctlr_el1, x6

isb

// 9. Jump to kernel resume entry point

br      x0       // x0 = KernelResumePhys

// Never returns

Section 6: Hibernate Image Invalidation After Resume

After successful resume, the hibernation image must be invalidated to prevent re-resuming from a stale state on the next reboot. ABL does this before jumping to the kernel as a safety measure:

// In ABL, after successful image load but BEFORE jumping to kernel

// Invalidate the header magic - if resume fails and device reboots,

// it will not attempt to resume from a partially-restored image

EFI_STATUS InvalidateHibernateImage(EFI_HANDLE PartHandle)

{

HIBERNATE_HEADER EmptyHeader;

SetMem(&EmptyHeader, sizeof(EmptyHeader), 0);

// Zero out ONLY the magic field to mark image as invalid

return WriteToPartition(PartHandle, 0,

sizeof(UINT32), &EmptyHeader.Magic);

}

// Kernel-side invalidation (after resume completes)

static void invalidate_hibernate_image(void)

{

struct hibernate_header hdr = {0};

write_to_partition("hibernate", 0, sizeof(hdr), &hdr);

sync_filesystem();

}

TWO-PHASE COMMIT SAFETY: The magic number in the header is written LAST after all page data is flushed. If power is lost during write, the magic number remains invalid and ABL falls back to normal cold boot on next power-on.

1.4  Key Challenges & Considerations

| Challenge | Problem | Solution |
| --- | --- | --- |
| DDR Training Consistency | DDR timing params must be identical to pre-hibernation values; temperature change can invalidate them | Save DDR training parameters alongside the hibernate image; force XBL to use them on resume path |
| Peripheral State | Peripherals (display, touch, audio) are in reset state after power-off | Kernel resume path re-initializes all peripherals; ABL can pre-init display for fast first-picture by using saved framebuffer content |
| Secure Boot Extension | Hibernate image = kernel memory dump; must be treated with same security as kernel itself | Sign with OEM key (same chain as boot.img); verify signature before restore; track rollback index separately |
| Image Size & Compression | 4GB RAM = up to 3GB hibernation image; naive implementation is too slow | LZ4 compression (~1 GB/s decompression on ARM); only save "dirty" pages; typical 2:1 to 3:1 ratio |
| Storage Speed | eMMC 5.1: ~300 MB/s (6.7s for 2GB). UFS 2.1: ~800 MB/s (2.5s). UFS 3.1: ~2.1 GB/s (0.95s) | UFS 3.1 storage required for sub-1-second restore; eMMC is too slow for automotive resume targets |

Storage Speed Impact on Resume Time (2GB Image)

| Storage Type | Read Speed | Time for 2GB | Automotive Viable? |
| --- | --- | --- | --- |
| eMMC 5.1 | ~300 MB/s | ~6.7s | NO |
| UFS 2.1 | ~800 MB/s | ~2.5s | Marginal |
| UFS 3.1 | ~2.1 GB/s | ~0.95s | YES |

1.5  ABL Boot Flow Decision with Hibernation

ABL Start

|

+-- Check for hibernate image

|

+-- Image found and valid?

|

+-- YES --> Verify signature

|               |

|               +-- Signature OK --> RESUME PATH:

|                                   1. Invalidate image header

|                                   2. Restore memory pages

|                                   3. (Optional) Init display

|                                      show saved framebuffer

|                                   4. Flush caches / TLB

|                                   5. Restore ARM64 CPU state

|                                   6. Jump to kernel resume

|

|               +-- Signature FAIL --> Invalidate image

|                                      --> Normal cold boot

|

+-- NO  --> Normal cold boot path:

1. Verified Boot (AVB)

2. Load kernel + ramdisk

3. Load DTB/DTBO

4. Jump to kernel _start

1.6  Cross-Questions: Hibernation in ABL

Q: Why not use the standard Linux hibernation resume through a restore kernel?

A: The standard approach requires booting a full restore kernel, which adds 1-2 seconds. In automotive, the requirement is often <2 seconds to display. By moving image restoration into ABL, we eliminate the restore kernel entirely. ABL reads the image directly from storage into DDR and jumps to the kernel's resume entry point, saving significant time.

Q: How do you handle the case where hardware changed between hibernate and resume?

A: This is a critical concern. We store hardware configuration metadata (DDR parameters, SoC revision, board ID) in the hibernate image header. On resume, ABL compares this against current hardware. If there's a mismatch (e.g., different DDR training results), ABL invalidates the image and falls back to normal cold boot. For DDR specifically, we save and restore the exact training parameters.

Q: What happens if power is lost during hibernation image write?

A: The image uses a two-phase commit approach. The magic number in the header is written last, after all page data is flushed. If power is lost during write, the magic number won't be valid, and ABL will do a normal boot. We also use CRC checksums to detect partial writes.

Q: How does this interact with Android Verified Boot (AVB)?

A: The hibernation image must be part of the verified boot chain. We sign the image with the same OEM key used for boot.img. ABL verifies the signature before restoring. Additionally, the rollback index for the hibernate image must be tracked separately to prevent rollback attacks where an attacker replaces the image with an older, vulnerable version.

Q: Can you resume from hibernation on a different device?

A: No. The hibernation image is tied to the specific device because: 1) It contains physical memory addresses specific to the DDR configuration, 2) Device-specific peripheral states are embedded, 3) The secure boot signature is device-specific. Attempting to restore on different hardware would either fail verification or cause an immediate kernel crash.

Q: Why do you need to disable the MMU before jumping to the kernel's resume entry?

A: ABL's own page tables (TTBR0_EL1, TTBR1_EL1) map virtual addresses for ABL's execution environment. The kernel's saved TTBR values map a completely different virtual address space. If we jump to the kernel with ABL's MMU settings active, the kernel's virtual address accesses would resolve to wrong physical addresses, causing immediate crashes. We disable the MMU, restore the kernel's saved TTBR values and SCTLR, then re-enable MMU with the kernel's page tables in place.

PART 2: BOOT TIME OPTIMIZATION - COMPLETE IN-DEPTH GUIDE

CRITICAL: "You can't optimize what you can't measure." - Always establish a precise measurement baseline before making any boot time changes. Every optimization must be validated with automated regression testing to prevent future regressions.

Boot time optimization matters across all embedded contexts:

- Automotive (IVI): Regulatory requirements - rear-view camera must display within 2 seconds of ignition.

- Consumer devices: User perception - every second of boot time impacts user satisfaction.

- Industrial/IoT: Fast recovery after power cycles in factory/field environments.

- Android CDD: Compatibility Definition Document recommends cold boot to launcher in <10 seconds.

2.1  Measurement Techniques (4 Methods)

Method 1: UART Timestamps (Most Common)

Enable timestamps in ABL and kernel command line to get per-stage timing from serial output:

# Enable timestamps in kernel command line:

androidboot.boottime=1

# ABL UART output with timestamps (example):

[0.000] PBL start

[0.052] PBL --> XBL handoff

[0.053] XBL start

[0.055] XBL: Clock init done

[0.180] XBL: DDR training start

[0.520] XBL: DDR training complete

[0.530] XBL --> ABL handoff

[0.535] ABL start

[0.600] ABL: Display init done

[0.720] ABL: AVB verification done

[0.780] ABL: Kernel loaded

[0.800] ABL: Jump to kernel

[0.810] Kernel: _start

[1.200] Kernel: init process started

Method 2: GPIO Toggle Method (Hardware-Level Precision)

Toggle a GPIO at each boot milestone. Measure with oscilloscope or logic analyzer for sub-millisecond accuracy, eliminating UART baudrate overhead:

// Toggle a GPIO at each boot milestone.

// Measure with oscilloscope or logic analyzer.

#define BOOT_MARKER_GPIO  42

void BootTimeMark(UINT32 Stage)

{

GpioToggle(BOOT_MARKER_GPIO);   // Rising edge on scope

MicroSecondDelay(10);            // 10us pulse width

GpioToggle(BOOT_MARKER_GPIO);   // Falling edge

}

// Usage:

BootTimeMark(1);  // PBL exit

BootTimeMark(2);  // DDR training complete

BootTimeMark(3);  // ABL start

BootTimeMark(4);  // Kernel entry

Method 3: Qualcomm Boot Profiling (SMEM-Based)

Read boot stage timings from SMEM (Shared Memory) after the device has fully booted. All stages write their timestamps to SMEM:

# After boot completes, read boot stats from SMEM via procfs:

$ adb shell cat /proc/boot_stats

PBL:          52ms

XBL:         478ms

ABL:         270ms

Kernel:      890ms

Init:       2100ms

Total:      3790ms

Method 4: Bootchart and Systrace (Android Init Phase)

For the Android init phase, bootchart and systrace provide detailed per-service timing and dependency graphs:

# Step 1: Enable bootchart

$ adb shell "touch /data/bootchart/enabled"

$ adb reboot

# Step 2: After boot, pull bootchart data

$ adb pull /data/bootchart/

$ java -jar bootchart.jar /data/bootchart/

# For systrace (Android init services):

$ python systrace.py -o trace.html --boot \

sched gfx view input audio

2.2  Stage 1: PBL Optimization (~50-100ms)

PBL is ROM code - you cannot modify it. However you can influence its behavior through eFuse configuration, partition layout, and image size reduction.

a) Boot Media Selection via eFuse Configuration

PBL tries multiple boot sources sequentially (eMMC -> SD -> USB), wasting time on failed attempts. Configure eFuses to specify the exact boot device:

Problem:  PBL tries multiple boot sources sequentially

eMMC --> SD Card --> USB (EDL mode)

Each failed attempt wastes 5-10ms

Solution: Blow BOOT_CONFIG fuses during manufacturing

to select eMMC/UFS only

Eliminates ~10-20ms of boot source probing

Qualcomm Tool: QFPROM (Qualcomm Fuse Blow ROM)

qpst_fuse.exe --fuse BOOT_CONFIG=0x2 (UFS only)

b) XBL Partition Placement in GPT

If the XBL partition is far into the eMMC, PBL takes longer to read the GPT and locate it. Place XBL in the first few LBAs after the GPT header:

Problem:  XBL partition at the end of GPT --> long seek time

# GPT layout optimization in partition.xml:

<partition label="xbl" size_in_kb="512" type="..."

readonly="true" slot_suffixes="ab"/>

# Place xbl immediately after protective MBR and GPT header

# LBA 0: Protective MBR

# LBA 1: GPT Header

# LBA 2-35: GPT Partition Table

# LBA 36: xbl_a partition <-- PLACE HERE

Savings: ~5-10ms on eMMC, less on UFS (sequential access)

c) XBL Image Size Reduction

- Remove unused drivers: WiFi, BT, USB if not needed in XBL/SBL (saves 50-200KB).

- Compiler flags: Use -Os (optimize for size) instead of -O2; every 100KB reduction ~= 0.3ms faster on UFS, ~1ms on eMMC.

- Strip debug strings: Remove DEBUG log strings in production builds.

2.3  Stage 2: XBL/SBL Optimization (~300-600ms) - BIGGEST OPPORTUNITY

XBL is where MOST boot time is spent, primarily due to DDR training. DDR fast boot (training parameter caching) is typically the SINGLE BIGGEST boot time optimization available.

Optimization A: DDR Training Fast Boot (200-400ms savings)

DDR training calibrates timing parameters for the DDR interface. These parameters are board-specific but stable across boots. Caching them eliminates 200-400ms:

FULL DDR TRAINING (~300-500ms per cold boot):

+------------------------------------------+

| Write Leveling        (~50ms)             |

| Read DQ Training      (~80ms)             |

| Read DQS Training     (~60ms)             |

| Write DQ Training     (~70ms)             |

| Vref Training         (~40ms)             |

| CA Training           (~30ms)             |

| Verification          (~20ms)             |

| TOTAL                 ~350ms              |

+------------------------------------------+

FAST BOOT - Training Restore (~40-80ms):

+------------------------------------------+

| Read saved params from flash  (~10ms)     |

| Apply params to DDR controller (~5ms)     |

| Quick verification R/W test   (~20ms)     |

| Validate against saved CRC    (~5ms)      |

| TOTAL                         ~40-80ms    |

+------------------------------------------+

SAVINGS: 250-400ms!

DDR Fast Boot Implementation

EFI_STATUS DdrInit(VOID)

{

DDR_TRAINING_PARAMS SavedParams;

EFI_STATUS Status;

// FAST PATH: Try loading saved training parameters

Status = LoadDdrTrainingParams(&SavedParams);

if (!EFI_ERROR(Status) &&

ValidateParamsCrc(&SavedParams)) {

DEBUG((EFI_D_INFO, "[DDR] Fast boot: restoring params\n"));

Status = ApplyDdrParams(&SavedParams);

if (!EFI_ERROR(Status)) {

// Quick verify: write pattern, read back

Status = DdrQuickVerify();

if (!EFI_ERROR(Status)) {

DEBUG((EFI_D_INFO, "[DDR] Fast boot SUCCESS\n"));

return EFI_SUCCESS;

}

}

DEBUG((EFI_D_WARN,"[DDR] Fast boot failed, full training\n"));

}

// SLOW PATH: Full training (first boot or fast boot failure)

DEBUG((EFI_D_INFO, "[DDR] Full DDR training starting\n"));

Status = DdrFullTraining(&SavedParams);

// Save parameters for next boot

if (!EFI_ERROR(Status)) {

SaveDdrTrainingParams(&SavedParams);

}

return Status;

}

When NOT to Use Fast Boot

- After firmware update: DDR controller code changed; saved params may not be compatible.

- Significant temperature change: >30C delta from training temperature; timing margins may have shifted.

- Extended storage: After device stored for weeks/months without power-on.

- Fast boot verification fails: Quick R/W test fails; automatically fallback to full training.

Optimization B: Parallel Initialization in XBL (50-100ms savings)

SEQUENTIAL (Before Optimization) - Total: 440ms:

[Clock Init 20ms] --> [PMIC Init 30ms] --> [DDR 350ms] --> [Storage 40ms]

PARALLEL (After Optimization) - Total: 370ms:

[Clock Init 20ms] --> +--- [PMIC Init 30ms]          ---+

+--- [DDR Training 350ms]      ---+

+--- [Storage Init 40ms]*      ---+

(* starts after PMIC sets voltage rails)

SAVINGS: ~70ms

// Qualcomm XBL DAL (Device Abstraction Layer) parallel init

void XblParallelInit(void)

{

// Start PMIC configuration (non-blocking)

PmicInitAsync();

// Start DDR training (long pole in the tent)

DdrTrainingStart();

// While DDR trains, initialize subsystems not needing DDR

ClockInitSecondary();    // Non-DDR clocks

GpioInit();              // GPIO configuration

CryptoEngineInit();      // For secure boot verification

// Wait for DDR training to complete

DdrTrainingWaitComplete();

// Now DDR is available - init storage (needs DDR for DMA)

StorageInit();

}

Optimization C: Hardware Crypto Engine (25-30ms savings)

Use Qualcomm's hardware Crypto Engine (CE) instead of software for RSA signature verification:

RSA-2048 Software verification:    ~30ms

RSA-2048 Hardware CE:              ~3ms   (10x speedup)

ECC-256  Software:                 ~10ms

ECC-256  Hardware CE:              ~1ms   (10x speedup)

# Enable hardware crypto in XBL config:

CONFIG_QCOM_CRYPTO_ENGINE=y

CONFIG_QCOM_SCM=y      # Secure Channel Manager for CE access

2.4  Stage 3: ABL (APPSBL) Optimization (~200-400ms)

Optimization A: Display Initialization (20-30ms savings)

SEQUENTIAL DISPLAY INIT (115ms):

[Panel detect 10ms] --> [DSI link 15ms] --> [Panel reset 50ms]

--> [Send init cmds 30ms] --> [Backlight ON 10ms]

OPTIMIZED PARALLEL (90ms):

[Panel detect 10ms] --> [DSI link + Reset PARALLEL (50ms)]

--> [Init cmds + Backlight PIPELINED (30ms)]

SAVINGS: ~25ms

// Key optimizations in display init code:

// 1. Reduce panel reset delay - test minimum viable time

//    Many panels specify 120ms but work with 50ms

// 2. Use continuous splash - pass framebuffer to kernel

//    Kernel inherits display state, no re-init needed

BootParamlist.ContinuousSplash = TRUE;

// 3. Pre-compute display config if hardware is fixed

//    Hardcode panel ID in CDT/DTB, skip runtime detection

Optimization B: Verified Boot (AVB) with Hardware Acceleration (30-60ms savings)

Standard AVB can take 80-120ms. With hardware crypto and read-hash pipelining, this can be reduced to 40-60ms:

STANDARD AVB (~80-120ms):

[Read vbmeta 5ms] --> [Verify sig 30ms SW] --> [Read header 2ms]

--> [Hash boot.img 50ms SW] --> [Compare 1ms]

OPTIMIZED AVB (~40-60ms):

[Read vbmeta 5ms] --> [Verify sig 3ms HW CE]

--> [Read+Hash pipeline using DMA 15ms HW CE]

SAVINGS: ~35-60ms

Read-Hash Pipeline Implementation

Overlap I/O and hashing: read next chunk while hashing current chunk, eliminating I/O wait during hashing:

EFI_STATUS PipelinedHashVerify(EFI_HANDLE Partition,

UINT64 Size)

{

UINT8 Buffer[2][CHUNK_SIZE];   // Double buffer

HASH_CTX HashCtx;

UINT32 ActiveBuf = 0;

UINT64 Offset = 0;

HashInit(&HashCtx, HASH_SHA256);

// Prime the pipeline: read first chunk

ReadFromPartitionAsync(Partition, Offset,

CHUNK_SIZE, Buffer[0]);

Offset += CHUNK_SIZE;

while (Offset < Size) {

UINT32 NextBuf = 1 - ActiveBuf;

// Start async DMA read of next chunk

ReadFromPartitionAsync(Partition, Offset,

CHUNK_SIZE, Buffer[NextBuf]);

// While reading, hash the current chunk via HW CE

HashUpdate(&HashCtx, Buffer[ActiveBuf], CHUNK_SIZE);

// Wait for read to complete

WaitForReadComplete();

ActiveBuf = NextBuf;

Offset += CHUNK_SIZE;

}

HashUpdate(&HashCtx, Buffer[ActiveBuf],

Size % CHUNK_SIZE);

HashFinal(&HashCtx, DigestOut);

return EFI_SUCCESS;

}

Optimization C: Kernel Loading with LZ4 Compression (30-50ms savings)

// LZ4 gives best read-time + decompress combined:

// Uncompressed Image: 30MB, read = 100ms (eMMC)

// LZ4 Image.lz4:      12MB, read = 40ms, decomp = 20ms

// Net savings:         40ms

# Kernel build config for LZ4:

CONFIG_KERNEL_LZ4=y

CONFIG_HAVE_KERNEL_LZ4=y

# In boot image build:

$ lz4 -l Image Image.lz4

$ mkbootimg --kernel Image.lz4 ...

Optimization D: Remove Debug Features in Production (20-40ms savings)

#ifdef PRODUCTION_BUILD

#define SKIP_FASTBOOT_CHECK    1 // Skip USB enum (10-20ms)

#define SKIP_CHARGER_DETECT    1 // Skip charger check (5-10ms)

#define SKIP_MDTP_CHECK        1 // Skip theft prot (5ms)

#define REDUCE_SPLASH_DELAY    1 // Reduce logo hold time

#define DISABLE_ABL_UART_LOGS  1 // UART is slow at 115200

#endif

2.5  Stage 4: Kernel Boot Optimization (~500-1000ms)

Optimization A: Kernel Compression Algorithm Comparison

| Algorithm | Compressed Size | Decompress Speed | Total Time (Read+Decomp) | Recommendation |
| --- | --- | --- | --- | --- |
| None (Image) | 30 MB | N/A | 100ms (eMMC) | Baseline |
| gzip (zImage) | 10 MB | ~200 MB/s | 33 + 150 = 183ms | Avoid - slow decomp |
| LZ4 (Image.lz4) | 12 MB | ~1.5 GB/s | 40 + 20 = 60ms | BEST CHOICE |
| LZO | 11 MB | ~800 MB/s | 37 + 37 = 74ms | Good alternative |
| ZSTD | 9 MB | ~500 MB/s | 30 + 60 = 90ms | Better ratio, slower |

Optimization B: Kernel Configuration Flags

# Disable unnecessary kernel features for faster boot:

CONFIG_PRINTK=n          # Disable kernel printk (50-100ms)

# Or use "quiet" in bootargs

CONFIG_KALLSYMS=n        # Remove symbol table (~200KB less)

CONFIG_DEBUG_INFO=n      # Remove debug info

CONFIG_MODULE_SIG=n      # Skip module signature verification

# Use built-in drivers for critical boot path (avoid module load)

CONFIG_MMC=y             # Built-in instead of module

CONFIG_USB=y             # Built-in instead of module

CONFIG_DISPLAY=y         # Built-in for continuous splash

# Kernel command line for fast boot:

console=null             # Disable console output (saves ~100ms)

loglevel=0               # Suppress all kernel messages

initcall_debug=0         # Disable initcall timing

rootwait                 # Wait for root device

Optimization C: Deferred Initialization (100-300ms savings)

// BEFORE: Critical path - all drivers init during boot

static int __init wifi_driver_init(void) { ... }

module_init(wifi_driver_init);  // Blocks boot

// AFTER: Defer non-critical drivers to late_initcall

static int __init wifi_driver_init(void) { ... }

late_initcall(wifi_driver_init); // Runs AFTER boot completes

// Even better: load as module on-demand

// WiFi not needed until user opens Settings app

// Initcall priority order:

// pure_initcall --> core_initcall --> postcore_initcall

// arch_initcall --> subsys_initcall --> fs_initcall

// device_initcall (module_init) --> late_initcall

2.6  Stage 5: Android Init Optimization (~2-4s) - Second Biggest Opportunity

Optimization A: Parallel Service Startup (500-700ms savings)

SEQUENTIAL DEFAULT (Total: 2200ms):

[mount 200ms] --> [SELinux 300ms] --> [Zygote 500ms]

--> [SysServer 800ms] --> [Launcher 400ms]

PARALLEL OPTIMIZED (Total: ~1500ms):

[mount 200ms] --> +--- SELinux (300ms)

+--- Zygote preload (500ms)

+--- SurfaceFlinger (100ms)

+--- ServiceManager (50ms)

After parallel phase:

SystemServer --> Launcher

SAVINGS: ~700ms

# In init.rc - mark services that can start in parallel:

service zygote /system/bin/app_process64 -Xzygote /

class main

socket zygote stream 660 root system

onrestart write /sys/android_power/request_state wake

# Parallel init trigger:

on zygote-start

# start multiple services concurrently

start surfaceflinger

start zygote

start zygote_secondary

Optimization B: Filesystem Optimization (100-200ms savings)

# Switch /data from EXT4 to F2FS (Flash-Friendly File System):

# F2FS has faster mount time and better random I/O for flash

# Switch system/vendor from EXT4 to EROFS:

# EROFS: Enhanced Read-Only File System

# - Faster mount time for read-only partitions

# - Compression support reduces partition size

# - Used by AOSP since Android 13

# dm-verity with hash prefetch:

veritytab system /dev/block/sda21 /dev/block/sda22 sha256 \

<roothash> <saltval> \

"prefetch_cluster 131072"    # Prefetch 128KB of hash tree

Optimization C: Zygote Preloading Optimization (100-200ms savings)

- Reduce preloaded classes: Only load what is needed for the launcher; defer framework classes.

- Usage-based profiles: Use class preloading profiles built from real usage data (ART profiles).

- Parallel resource loading: Preload resources in parallel with class loading using multiple threads.

- Pre-compiled DEX: Avoid JIT compilation on first boot; deliver pre-compiled .oat files.

2.7  Summary Optimization Table

| Stage | Optimization | Savings | Difficulty | Risk | Priority |
| --- | --- | --- | --- | --- | --- |
| PBL | Boot source eFuse config | 10-20ms | Low | Low | Medium |
| PBL | XBL partition placement in GPT | 5-10ms | Low | Low | Low |
| XBL | DDR training restore (fast boot) | 200-400ms | Medium | Medium | CRITICAL |
| XBL | Parallel HW initialization | 50-100ms | Medium | Medium | High |
| XBL | Hardware crypto engine for AVB | 25-30ms | Low | Low | High |
| XBL | XBL image size reduction | 5-20ms | Low | Low | Medium |
| ABL | Display init optimization | 20-30ms | Medium | Low | High |
| ABL | AVB hardware crypto acceleration | 30-60ms | Medium | Low | High |
| ABL | Read-hash pipeline for kernel | 20-40ms | High | Low | Medium |
| ABL | LZ4 kernel compression | 30-50ms | Low | Low | High |
| ABL | Remove debug features (production) | 20-40ms | Low | Medium | High |
| Kernel | LZ4 compression (kernel config) | 40-120ms | Low | Low | High |
| Kernel | Quiet boot (no printk) | 50-100ms | Low | Medium | High |
| Kernel | Deferred driver init (late_initcall) | 100-300ms | Medium | Medium | High |
| Init | Parallel service startup | 500-700ms | High | High | CRITICAL |
| Init | F2FS filesystem for /data | 100-200ms | Medium | Medium | High |
| Init | Zygote preloading optimization | 100-200ms | High | Medium | Medium |

Cumulative potential savings: 1.5 - 3 seconds  (from ~5-6s down to ~2-3s)

2.8  Real-World Before/After Example

BEFORE OPTIMIZATION:

+-------------------------------------------------------+

| PBL: 80ms | XBL: 520ms | ABL: 350ms | Kernel+Init: 4.2s|

| TOTAL: 5.15 seconds to home screen                    |

+-------------------------------------------------------+

AFTER OPTIMIZATION:

+-------------------------------------------------------+

| PBL: 65ms | XBL: 180ms | ABL: 190ms | Kernel+Init: 2.1s|

| TOTAL: 2.54 seconds to home screen                    |

+-------------------------------------------------------+

KEY CHANGES APPLIED:

XBL:    DDR fast boot                 -340ms saved

Parallel init                  -50ms saved

Hardware crypto                -15ms saved

ABL:    HW crypto for AVB              -30ms saved

LZ4 kernel (read+decomp)       -40ms saved

Display init optimization       -25ms saved

Removed debug features         -40ms saved

Kernel: Quiet boot (no printk)         -80ms saved

Deferred driver init          -200ms saved

Init:   Parallel services             -600ms saved

F2FS filesystem               -150ms saved

TOTAL SAVED: 2.57 seconds

2.9  Cross-Questions: Boot Time Optimization

Q: What is the single biggest boot time optimization you can make?

A: DDR training parameter caching (fast boot). It saves 200-400ms, which is typically the single largest contributor to boot time in the XBL stage. The second biggest opportunity is parallelizing Android init services, which saves 500-700ms. Together these two optimizations alone can cut more than a second from total boot time.

Q: How do you balance boot time optimization vs reliability?

A: Every optimization has a risk profile. DDR fast boot saves the most time but has a risk of using stale parameters - we mitigate with quick verification and automatic fallback to full training. Disabling kernel printk saves time but makes debugging harder - keep it in engineering builds, disable in production only. Parallel init can hide race conditions - requires thorough testing. The key is: always have a safe fallback path for every optimization.

Q: How do you measure boot time accurately?

A: Multiple methods: 1) UART timestamps for software-level timing in each stage, 2) GPIO toggling with oscilloscope for hardware-level precision eliminating UART overhead, 3) Qualcomm SMEM boot stats for stage-level breakdown after boot, 4) bootchart for Android init phase profiling. Always use at least two methods to cross-validate and detect measurement artifacts.

Q: What if the customer wants sub-1-second boot time?

A: Sub-1-second cold boot is extremely challenging. Options: 1) Hibernation/suspend-to-disk (ABL-based resume as covered in Part 1 of this document), 2) Snapshot boot - pre-built system image loaded directly, 3) Minimal Linux with custom init - skip Android entirely for first display frame, 4) FPGA-assisted display - show camera feed from hardware before CPU boots. For automotive rear-view camera, option 4 is common: hardware displays camera within 500ms while Linux boots in background.

Q: How do you prevent boot time regression in CI/CD?

A: Automated boot time CI/CD pipeline: 1) Every firmware build triggers automated boot time measurement on reference hardware, 2) GPIO-based measurement for hardware-level consistency across test runs, 3) Threshold alerts - if any stage exceeds its time budget by more than 10%, the build is flagged for review, 4) Boot time budget document where each team owns their stage's time budget and must justify any increase. Never allow "I'll fix it later" for boot time regressions.

Q: What is the difference between cold boot and warm boot optimization?

A: Cold boot starts from power-off (PBL -> full sequence, DDR must be trained). Warm boot (reboot) can skip some hardware initialization because DDR is already trained and PMIC is configured - typically 30-40% faster. Cold boot optimization focuses on DDR training and hardware init; warm boot focuses on kernel and Android init. Suspend/resume (S3) is even faster - only restores CPU state from RAM, no storage I/O needed.

Q: How does A/B partition scheme affect boot time?

A: A/B adds 10-20ms overhead: ABL must read the boot control block to determine active slot, and may try the other slot on failure. However, A/B eliminates the need for a separate recovery partition and enables seamless OTA. To minimize impact, cache the active slot in SMEM to avoid re-reading on each boot. The reliability benefit outweighs the small time cost.

Q: When should you NOT use DDR fast boot (training restore)?

A: Avoid fast boot when: 1) After firmware update - DDR controller code may have changed, making saved parameters incompatible, 2) After significant temperature change (>30C delta) - timing margins shift with temperature, 3) After device has been in storage for an extended period, 4) If the quick verification test fails - the quick R/W test specifically detects if saved parameters are still valid. Always implement automatic fallback to full training when fast boot fails.

2.10  Interview Talking Points - Sample Answer Framework

Sample interview answer combining your experience:

"On our SDM845 automotive platform, I led a boot time optimization effort that reduced cold boot from 5.2 seconds to 2.4 seconds. The biggest wins were DDR training parameter caching (saved 340ms), parallelizing XBL initialization (saved 50ms), switching to LZ4 kernel compression (saved 40ms), and implementing hardware crypto acceleration for AVB (saved 30ms). On the Android side, we parallelized init services and switched to F2FS, saving another 750ms. We set up an automated GPIO-based measurement CI pipeline to prevent regressions."

Key Terms to Use in Interview

| Boot Time Terms | Tools & Methods |
| --- | --- |
| - DDR training restore / fast boot - Parallel initialization - Continuous splash / framebuffer handoff - LZ4 / ZSTD compression tradeoffs - Deferred initialization / late_initcall - Read-hash pipeline / DMA overlap | - UART timestamps / SMEM boot stats - GPIO toggle + oscilloscope - bootchart / systrace - Lauterbach Trace32 JTAG - Automated boot time CI regression - Qualcomm QFPROM fuse configuration |